ANNEXURE - VI

THAPAR INSTITUTE OF ENGINEERING & TECHNOLOGY, PATIALA

Department of Electronics & Communication Engineering

PROJECT SEMESTER: 2025-26 Even Semester (from January 2026)

FINAL INTERNSHIP REPORT

(Second Visit)

Student Name: Reyansh

Roll Number: 102206005

Organisation: Arm Ltd., Bangalore

Department: CPU Verification Group - Interconnect Team

Duration: January 2026 - June 2026

Date of Visit: June 2026

Submitted in partial fulfilment of the requirements

for the degree of Bachelor of Engineering

in Electronics & Communication Engineering

TABLE OF CONTENTS

1. Introduction

2. Internship Details

3. Organisation Overview

4. SoC Architecture and System Understanding

5. Learning Phase

6. Project 1: Status Register Checker

7. Project 2: Compile Warning Resolution

8. Project 3: CMN Clock Controller Integration

9. Project 4: AI Agent for Hardware Programming Code Generation

10. Results and Outcomes

11. Major Challenges and Innovations

12. Skills Acquired

13. Conclusion

14. References

# 1. Introduction

This report presents the complete technical work accomplished during my six-month internship at Arm Ltd., Bangalore, from January 2026 to June 2026. The internship was placed within the CPU Verification Group, specifically the Interconnect Team, which is responsible for verifying the functional correctness and protocol compliance of on-chip interconnect fabrics used in Arm's next-generation processor designs.

Modern System-on-Chip (SoC) designs are extraordinarily complex, integrating billions of transistors across multiple CPU cores, GPU compute units, neural processing units, memory controllers, and hundreds of peripheral interfaces. The interconnect fabric -- the communication backbone connecting all components -- must operate with absolute correctness under all possible traffic patterns, coherency scenarios, and timing conditions. A single undetected bug can cause system-wide data corruption or security vulnerabilities that may not manifest until millions of chips are manufactured.

During this internship, I contributed four distinct projects:

- Project 1: Automated Status Register Checker for cycle-accurate RTL-to-model comparison.

- Project 2: Systematic compile warning resolution achieving zero-warning compilation.

- Project 3: Clock controller integration into the Coherent Mesh Network (CMN).

- Project 4: AI-powered agent for automated hardware programming code generation from new RTL.

Each project required different skills -- from SystemVerilog/UVM methodology through Python automation to AI/LLM capabilities. Together, they represent a comprehensive contribution to Arm's verification infrastructure.

# 2. Internship Details

Organisation: Arm Ltd., CPU Verification Group - Interconnect Team, Bangalore

Duration: January 2026 - June 2026 (Project Semester 2025-26)

Team Size: 12 engineers (Interconnect Verification Team)

Location: Arm Development Centre, Outer Ring Road, Bangalore

## 2.1 Internship Timeline

| Month | Phase | Key Activities |
| --- | --- | --- |
| Jan 2026 | Onboarding | ARM architecture, UVM training, tool setup |
| Feb 2026 | Learning + P1 | Protocol deep-dive, Status Register Checker |
| Mar 2026 | P1 + P2 | Checker completion, warning analysis |
| Apr 2026 | P2 + P3 | Warnings resolved, CMN clock controller |
| May 2026 | P3 + P4 | Clock controller verified, AI agent begins |
| Jun 2026 | P4 + Closure | AI agent complete, documentation |

## 2.2 Tools and Technologies

| Category | Tool | Purpose |
| --- | --- | --- |
| HDL | SystemVerilog (IEEE 1800-2017) | RTL and verification |
| Verification | UVM 1.2 | Testbench methodology |
| Scripting | Python 3.10 | Automation, AI agent |
| Simulation | Synopsys VCS / Cadence Xcelium | RTL simulation |
| Waveform | Synopsys Verdi | Debug and analysis |
| Ref Model | Arm Fast Model | Architectural simulation |
| VCS | Git + Gerrit | Code management and review |
| AI/ML | LLM APIs, Python | Code generation |

# 3. Organisation Overview - Arm Ltd.

Arm Ltd. is the world's leading semiconductor IP company, designing processor architectures that power over 99% of smartphones and a growing share of servers, automotive, and IoT devices. Arm licenses processor designs to partners (Apple, Qualcomm, Samsung, NVIDIA) who integrate them into their SoCs.

A bug in Arm's IP can propagate to billions of devices. This makes verification the most critical engineering discipline in the company, and is the function where this internship was placed.

## 3.1 The Interconnect Team

The Interconnect Team verifies the communication fabric connecting CPU cores, memory, and peripherals. Key responsibilities:

- Protocol compliance verification (AXI4, ACE, CHI, APB)

- Cache coherency correctness in multi-core configurations

- Power management interface verification (clock gating, domain transitions)

- Performance verification (throughput and latency targets)

- Regression maintenance (tens of thousands of nightly tests)

# 4. SoC Architecture and System Understanding

Understanding the complete SoC architecture was essential for all four projects. A modern Arm-based SoC integrates the following major subsystems:

Figure 1: High-Level SoC Architecture Block Diagram

| CPU Cluster (Cortex-A/X) | GPU (Mali) | Interconnect (CMN) | Memory Ctrl (DDR5) |
| --- | --- | --- | --- |
| System MMU | DMA Controller | Peripheral Bus (APB) | I/O Interfaces (USB, PCIe) |
| Debug & Trace (CoreSight) | Power Mgmt (PMU) | Neural Engine (NPU) | Security (TrustZone) |

## 4.1 CPU Subsystem

Figure 2: CPU Subsystem - Heterogeneous Core Configuration

CPU Cluster (DynamIQ)

↓  ↓  ↓

| Cortex-X4 (Performance) | Cortex-A720 x3 (Balanced) | Cortex-A520 x4 (Efficiency) | Shared L3 (SLC) |
| --- | --- | --- | --- |

Each core has private L1 caches (64KB each), shared L2 (512KB-2MB), and cluster-wide L3 (4-16MB) managed by the Coherent Mesh Network. Coherency is maintained via CHI snoop protocol.

## 4.2 Interconnect Fabric

Figure 3: AMBA Protocol Hierarchy (Highest to Lowest Complexity)

CHI - Coherent Hub Interface (Full cache coherency, multi-core)

↓

ACE - AXI Coherency Extensions (Adds coherency to AXI)

↓

AXI4 - Advanced eXtensible Interface (High bandwidth, pipelined)

↓

AHB - Advanced High-performance Bus (Simpler, single channel)

↓

APB - Advanced Peripheral Bus (Low bandwidth, registers)

## 4.3 Memory Subsystem

Figure 4: Memory Subsystem Data Path

System Level Cache (L3 / LLC)

↓

Memory Controller (Command Scheduling)

↓

DDR PHY (Physical Interface Layer)

↓

External DDR5 / LPDDR5 DRAM

# 5. Learning Phase

The first six weeks were dedicated to building the technical foundation for productive contribution.

## 5.1 ARMv8-A Architecture

- Exception Levels (EL0-EL3): Privilege hierarchy from user (EL0) to secure monitor (EL3).

- Register File: 31 general-purpose 64-bit registers (X0-X30), SP, PC, PSTATE.

- System Registers: Control cache, MMU, exceptions, debug, performance monitoring.

- Memory Model: Weakly-ordered with explicit barriers (DMB, DSB, ISB).

## 5.2 AXI4 Protocol

| Channel | Direction | Purpose | Key Signals |
| --- | --- | --- | --- |
| AW | Master->Slave | Write address | AWADDR, AWLEN, AWSIZE |
| W | Master->Slave | Write data | WDATA, WSTRB, WLAST |
| B | Slave->Master | Write response | BRESP, BID |
| AR | Master->Slave | Read address | ARADDR, ARLEN, ARSIZE |
| R | Slave->Master | Read data | RDATA, RRESP, RLAST |

Figure 5: AXI4 - Independent Write and Read Paths

| AW (Write Address) | ➡ | AR (Read Address) |
| --- | --- | --- |
| W (Write Data) | ➡ | R (Read Data) |
| B (Write Response) | ➡ |  |

- Outstanding Transactions: Multiple addresses before first response (hides latency).

- Out-of-Order Completion: Different IDs complete independently.

- Burst Transfers: Up to 256 beats per address phase.

## 5.3 APB Protocol

Figure 6: APB Transfer State Machine

IDLE - No transfer active

↓

SETUP - PSEL=1, PENABLE=0

↓

ACCESS - PSEL=1, PENABLE=1

↓

Complete - PREADY=1, data valid

## 5.4 CHI Protocol

| Node | Full Name | Role | Example |
| --- | --- | --- | --- |
| RN-F | Request Node Full | Snoopable cache | CPU core |
| RN-I | Request Node I/O | Non-snoopable | DMA, GPU |
| HN-F | Home Node Full | Coherency point | Interconnect |
| SN-F | Subordinate Node | Memory provider | DDR ctrl |
| MN | Misc Node | System functions | Debug |

Figure 7: CHI Node Topology in Multi-Core System

| RN-F (Core 0) | RN-F (Core 1) | RN-F (Core 2) |
| --- | --- | --- |
| HN-F (Home) | SN-F (Memory) | RN-I (DMA) |

CHI uses four channels: REQ (requests), RSP (responses), DAT (data), SNP (snoops). Implements MOESI cache coherency across all nodes.

## 5.5 UVM 1.2 Methodology

Figure 8: UVM Testbench Architecture

UVM Test Environment

↓  ↓  ↓

| Agent (Driver+Monitor) | Scoreboard (Comparison) | Coverage (Metrics) | Sequencer (Stimulus) |
| --- | --- | --- | --- |

| Component | Base Class | Function |
| --- | --- | --- |
| Agent | uvm_agent | Encapsulates driver + monitor + sequencer |
| Driver | uvm_driver | Converts transactions to pin signals |
| Monitor | uvm_monitor | Passively observes DUT |
| Scoreboard | uvm_scoreboard | Compares expected vs actual |
| Sequence | uvm_sequence | Defines stimulus patterns |

Figure 9: UVM Agent Data Flow Pipeline

Sequencer (routes sequence items)

↓

Driver (converts to pin-level signals)

↓

DUT (Design Under Test)

↓

Monitor (passive observation via interface)

↓

Scoreboard (expected vs actual comparison)

## 5.6 Coherent Mesh Network (CMN)

Figure 10: CMN 4x4 Mesh Topology (Cross-Point Grid)

| XP+RN-F | XP+RN-F | XP+HN-F | XP+SN-F |
| --- | --- | --- | --- |
| XP+RN-F | XP+RN-F | XP+HN-F | XP+SN-F |
| XP+RN-I | XP+SBSX | XP+HN-I | XP+CCG |

| Node | Full Name | Function |
| --- | --- | --- |
| XP | Cross-Point | Mesh router, routes flits by target ID |
| RN-F | Request Node | CPU interface with full coherency |
| HN-F | Home Node | Point-of-Coherence, SLC slice |
| SN-F | Subordinate | Memory controller interface |
| SBSX | System Bridge | Bridge to external AXI |
| CCG | Cross-Chip Gateway | Multi-chip link (CCIX/CXL) |
| DTC | Debug Trace Ctrl | Debug data collection |

Figure 11: CMN Cross-Point Internal Architecture

Cross-Point (XP) Router

↓  ↓  ↓

| N/S/E/W Mesh Links | Device Port 0 | Device Port 1 | Arbiter + Routing |
| --- | --- | --- | --- |

Each XP performs flit buffering, round-robin arbitration, dimension-ordered routing (X then Y), and credit-based flow control. SLC is distributed across HN-F nodes with address interleaving.

# 6. Project 1: Status Register Checker

An automated checker for a CPU debug/trace subsystem's status register, performing cycle-accurate comparison between RTL and the architectural reference model. The checker identified a pre-existing bug undetected in manual review.

Figure 12: Status Register Checker - Complete Data Flow

Arm Fast Model (Architectural Reference Simulation)

↓

Log File Output (Multi-GB, streaming required)

↓

Python Log Parser (Compiled regex, O(1) lookup)

↓

sim_opts Transfer (Command-line to SystemVerilog)

↓

UVM Scoreboard (Timestamp-keyed comparison)

↓

Pass/Fail Reporting (uvm_fatal on mismatch)

## 6.1 Python Log Parser

The Arm Fast Model generates multi-GB logs. The parser streams line-by-line with compiled regex for constant memory footprint (~10MB regardless of file size).

Figure 13: Python Log Parser - Processing Pipeline

Open log file (streaming mode)

↓

Read line-by-line (constant memory)

↓

Apply compiled regex (pattern matching)

↓

Extract register value + timestamp

↓

Store in dict keyed by time (O(1))

↓

Pass to simulation via sim_opts

- Memory Efficiency: Flat footprint regardless of file size (~500 MB/s throughput).

- Robustness: Try-except recovery on malformed lines, logs warning and continues.

- Configurability: Log path, pattern, format all command-line configurable.

## 6.2 UVM Scoreboard

Figure 14: UVM Scoreboard - Dual Input Architecture

| Expected Queue (from sim_opts) | ➡ | Actual Queue (from RTL Monitor) |
| --- | --- | --- |
| Timestamp Key (assoc. array) | ➡ | TLM Analysis Port |
| Bit-Field Spec | ➡ | Real-Time Compare |

- Step 1: Monitor publishes register write via TLM analysis port.

- Step 2: Scoreboard extracts simulation timestamp.

- Step 3: Looks up expected value by timestamp (O(1) associative array).

- Step 4: Full 32-bit equality (=== catches X/Z). Match = UVM_HIGH log. Mismatch = uvm_fatal.

- Integration via UVM Factory override -- zero modification to existing code.

## 6.3 Timestamp Synchronisation

Figure 15: Timestamp Reordering Problem (Model vs RTL order differs)

| Model: Event A @ T=100 | ➡ | RTL: Event B @ T=102 |
| --- | --- | --- |
| Model: Event B @ T=100 | ➡ | RTL: Event A @ T=105 |
| Model: Event C @ T=200 | ➡ | RTL: Event C @ T=201 |

Solution: Timestamp-keyed dictionary lookup with configurable tolerance window (5 cycles). Eliminated all false positives while correctly detecting genuine mismatches.

Goal: Zero-false-positive automated register comparison in live regression.

# 7. Project 2: Compile Warning Resolution

Arm mandates zero-warning compilation. Systematically identified and resolved all 153 warnings across 47 source files in 8 regression configurations.

## 7.1 Warning Categories

| Category | Count | % | Risk Level |
| --- | --- | --- | --- |
| Unused Variables/Signals | 47 | 31% | Low-Medium |
| Implicit Type Conversions | 34 | 22% | Medium-High |
| Deprecated Constructs | 28 | 18% | Low |
| Uninitialised Signals | 23 | 15% | High |
| Port Width Mismatches | 14 | 9% | High |
| Unreachable Code | 7 | 5% | Low |

Figure 17: Warning Category Distribution

| Unused Vars 47 (31%) | Type Convert 34 (22%) | Deprecated 28 (18%) |
| --- | --- | --- |
| Uninitialised 23 (15%) | Port Width 14 (9%) | Unreachable 7 (5%) |

## 7.2 Resolution Methodology

Figure 16: Six-Step Warning Resolution Workflow

1. Identify Warning Source (parse log, group by root cause)

↓

2. Understand Original Intent (git blame, context)

↓

3. Determine Correct Fix (most conservative approach)

↓

4. Apply Minimum Change (no opportunistic cleanup)

↓

5. Run Targeted Regression (affected tests)

↓

6. Full Regression Validation (overnight suite)

Critical lesson: An early port-width fix truncated a connection incorrectly (port declaration was the error, not the connection). Caused a regression failure 3 days later. Added mandatory downstream trace analysis for all width fixes.

Goal: Zero-warning compilation across all configurations.

# 8. Project 3: CMN Clock Controller Integration

Integration of a clock controller into the Coherent Mesh Network for dynamic power management. Involved RTL integration, clock domain crossing verification, and power management protocols.

## 8.1 Clock Controller Architecture

Figure 18: Clock Controller - Block Diagram

CMN Clock Controller

↓  ↓  ↓

| Clock Dividers | Gate Control | Wake-up Logic | Q-Channel I/F | Status Regs |
| --- | --- | --- | --- | --- |

| Clock Domain | Components | Gating Policy |
| --- | --- | --- |
| Mesh Clock | XP routers, links | Gate when no flits in flight |
| Device Clock | RN-F, HN-F, SN-F | Per-node based on activity |
| SLC Clock | Cache arrays | Gate when no lookups pending |
| Debug Clock | DTC, debug logic | Independent control |
| Always-On | Wake-up, sync regs | Never gated |

## 8.2 State Machine

Figure 19: Clock Controller Power State Machine

RUNNING (Full clock active, processing transactions)

↓

GATE_REQUEST (Draining in-flight transactions)

↓

CLOCK_GATED (No clock delivered to node)

↓

WAKE_REQUEST (Snoop/DMA/SW wake-up received)

↓

RESTORING (Clock ramping, PLL stabilising)

↓

RUNNING (Back to active, ready for traffic)

## 8.3 Clock Domain Crossing Verification

Figure 20: Clock Domain Crossing Architecture

| Always-On Domain (wake-up logic) | ➡ | 2-FF Synchroniser (control signals) |
| --- | --- | --- |
| Handshake Protocol (req/ack) | ➡ | Async FIFO (data path) |
| Gray-Code FIFO (pointer sync) | ➡ | Gated Domain (CMN node) |

- Structural CDC analysis: All 14 crossing paths verified for synchroniser presence.

- Functional CDC: Stress tests targeting simultaneous state transitions.

- Formal properties: SVA assertions on all handshake protocols.

## 8.4 Results

- Two RTL bugs found: (1) missing synchroniser on wake-up ack, (2) incorrect snoop priority during gating.

- 100% state transition coverage achieved in directed tests.

- All 14 CDC paths confirmed correctly synchronised.

Goal: Correct clock controller integration with zero functional impact on CMN.

# 9. Project 4: AI Agent for Hardware Programming Code Generation

An AI-powered agent that automatically generates hardware programming code whenever new RTL is delivered, reducing the RTL-to-verification timeline from days to hours.

## 9.1 Problem

When new RTL arrives, engineers manually write register init sequences, mode programming, test setup, power management, and clock configuration code. This takes 2-3 days per block, is error-prone, and creates a bottleneck.

## 9.2 Solution Architecture

Figure 21: AI Agent - Complete System Architecture

Input: New RTL Source Files (.sv/.v)

↓

Stage 1: RTL Parser (Extract registers, ports, hierarchy)

↓

Stage 2: Context Builder / RAG (Find similar code, specs)

↓

Stage 3: LLM Code Generator (Structured prompts + constraints)

↓

Stage 4: Validator (Address/width/access-type checks)

↓

Output: Programming Code (Ready for engineer review)

## 9.3 RTL Parsing Pipeline

Figure 22: RTL Parsing Pipeline

SystemVerilog source file

↓

Lexer / Tokenizer

↓

Module + Port extraction

↓

Register definition parser

↓

Structured intermediate repr. (JSON)

- Module interface extraction: ports, widths, directions, types.

- Register map: address offsets, fields, bit positions, access types, reset values.

- Incremental: only re-parses changed files.

## 9.4 Integration Workflow

Figure 23: Code Generation - CI/CD Integration Flow

Engineer commits new RTL

↓

CI/CD triggers agent automatically

↓

Agent parses RTL, builds context

↓

LLM generates programming code

↓

Automated validation runs

↓

Code placed in review branch

↓

Engineer reviews and approves

↓

Merged to main codebase

## 9.5 Results

| Metric | Before (Manual) | After (AI Agent) | Improvement |
| --- | --- | --- | --- |
| Time to first code | 2-3 days | < 1 hour | 10-20x faster |
| Style consistency | Variable | Uniform | Standardised |
| Register address errors | 2-3 per block | 0 (validated) | Eliminated |
| Engineer effort | Write from scratch | Review only | Focus on logic |

Goal: 10x reduction in RTL-to-verification timeline via automated code generation.

# 10. Results and Outcomes

## 10.1 Project 1 - Status Register Checker

- Integrated into live regression, found pre-existing bug.

- Zero false positives via timestamp-keyed lookup.

- Functional coverage bins for all status bit combinations.

## 10.2 Project 2 - Compile Warnings

- Zero-warning compilation across all 8 configurations.

- 153 warnings resolved across 47 files. One latent bug exposed.

## 10.3 Project 3 - Clock Controller

- Two RTL bugs found (missing synchroniser, priority error).

- 100% state transition coverage. All 14 CDC paths verified.

## 10.4 Project 4 - AI Agent

- 10-20x faster than manual. Zero address errors.

- 3 RTL blocks successfully processed in final month.

# 11. Major Challenges and Innovations

## 11.1 Timestamp Race (P1)

Problem: Model/RTL produce events in different orders. Solution: Timestamp-keyed dictionary with tolerance window.

## 11.2 Multi-GB Logs (P1)

Problem: 5-10GB on memory-constrained servers. Solution: Streaming + compiled regex = constant 10MB footprint.

## 11.3 Warning Safety (P2)

Problem: Fixes risk functional changes. Solution: 6-step methodology with mandatory downstream analysis.

## 11.4 CDC Bugs (P3)

Problem: 14 clock crossings, timing-dependent bugs. Solution: Structural + functional + formal analysis. Found critical missing synchroniser.

## 11.5 Code Accuracy (P4)

Problem: Undocumented register layouts. Solution: Multi-source extraction (RTL parse + specs + pattern matching).

## 11.6 Non-Disruptive Integration (P1)

Problem: Stable testbench, can't modify. Solution: UVM factory override = zero source changes, per-test enable/disable.

# 12. Skills Acquired

## 12.1 Technical

- Industrial UVM verification (100K+ line testbenches)

- ARM protocol expertise (AXI4, APB, CHI, ACE)

- SoC architecture (CPU, interconnect, memory, peripherals)

- CMN architecture (mesh, routing, SLC, clock distribution)

- CDC design and verification (synchronisers, formal)

- Power management (clock gating, Q-Channel)

- Python automation (log parsing, regression, code gen)

- AI/LLM for hardware (prompts, RAG, validation)

- SystemVerilog (IEEE 1800-2017, Arm standards)

## 12.2 Professional

- Multi-team codebases with Git + Gerrit code review

- Agile practices (standups, sprints, retros)

- Technical communication (design reviews, documentation)

- Cross-timezone collaboration

# 13. Conclusion

This six-month internship produced four significant contributions: an automated register checker, zero-warning baseline, CMN clock controller integration, and an AI code generation agent. All deliverables met Arm's quality standards and are in production use.

The experience provided deep exposure to SoC design and verification -- from architecture through protocols to AI tooling. The combination of traditional verification and modern AI-assisted development reflects the evolving landscape of hardware engineering.

(Signature of Faculty Mentor)                    (Signature of Industry Mentor)

Name...............................           Name...................................

Designation........................           Designation............................

# 14. References

[1] ARM, AMBA AXI and ACE Protocol Specification, ARM IHI 0022H, 2021.

[2] ARM, AMBA APB Protocol Specification, ARM IHI 0024E, 2021.

[3] ARM, AMBA CHI Architecture Specification, ARM IHI 0050G, 2022.

[4] ARM, ARM Architecture Reference Manual ARMv8, ARM DDI 0487J, 2023.

[5] ARM, CoreLink CMN-700 TRM, ARM DDI 0622, 2022.

[6] IEEE, SystemVerilog Standard, IEEE 1800-2017.

[7] Accellera, UVM 1.2 User's Guide, 2014.

[8] C. Spear, SystemVerilog for Verification, Springer, 3rd Ed, 2012.

[9] ARM, CoreSight Architecture Specification, ARM IHI 0029E, 2021.

[10] ARM, Q-Channel and P-Channel Interfaces, ARM IHI 0068D, 2020.

[11] C. Cummings, CDC Design & Verification Techniques, SNUG 2008.

[12] J. Bergeron, Writing Testbenches, Springer, 2003.

[13] ARM, AMBA Low Power Interface Spec, ARM IHI 0051B, 2019.

[14] B. Wile et al, Comprehensive Functional Verification, Morgan Kaufmann, 2005.

[15] ARM, CoreLink CMN-S3 TRM, ARM DDI 0630, 2023.